S E C T I O N  7A

STREETS - DESIGN

7A.l	Scope

7A.2	Width of Right-of-Way

7A.3	Width of Pavement

7A.4	Design Cross Sections

7A.5	Street Classification Definitions

7A.6	Minimum Base and Pavement Strengths

7A.7	Standard Longitudinal Grade

7A.8	Standard Crown and Crossfall Grades

7A.9	Geometrics

7A.l0	Earthwork

7A.ll	Standard Cross Sections and Utilities

7A.12	Cul-de-sacs

7A.l3	Temporary Cul-de-sacs

7A.l4	Driveways

7A.l5	Street Name and Traffic Signs

7A.l6	Street Markings

7A.17	Community Mail Centres

7A.18	Half Road (Amendment Bylaw 1932)

STANDARD DRAWINGS

R1	Downtown

R2	Urban Collector with Parking

R2C	Urban Collector (Amendment Bylaw 1932)

R3	Rural/Industrial Collector

R4	Urban Local

R5	Rural/Industrial Local

R6	Lane

R7	Standard Cul-de-sac and Offset Cul-de-sac

R8	Rural/Industrial Cul-de-sac and Offset Cul-de-sac

R9	Statutory Right of Ways for Municipal Services

R10	Typical Driveway Grades

R11	Typical Sign Installations

S E C T I O N 7A

STREETS - DESIGN

7A.l	SCOPE

In general, streets shall be designed in accordance with the "Manual of Geometric Design Standards for Canadian Roads and Streets", distributed by the Transportation Association of Canada (TAC).

7A.2	WIDTH OF RIGHT-OF-WAY

When approved by the Engineer, the right-of-way widths listed below may be varied.

7A.2.l	Road Class				R/W Width	Std. Dwg. No.

Downtown (Parking Both Sides)		24.0m		R1

Urban Collector 				20.0 m 		R2

Rural/Industrial Collector 			21.0 m		R3

Urban Local (Parking Both Sides)		15.5m		R4

Rural/Industrial Local 			18.0 m		R5

Lane					6.0m		R6

Urban Cul-de-sac Diameter		30.0 m		R7

Rural/Industrial Cul-de-sac Diameter	36.0m		R8

7A.2.2	In hillside locations where the land slope exceeds ten percent (l0%) the minimum width of right-of-way shall be 20 metres.

7A.3	WIDTH OF PAVEMENT (Gutter to Gutter)

When approved by the Engineer, the pavement widths listed below may be varied.

Road Class			Pavement Width		Dwg. No.

Downtown			19.0 m			R1

Urban Collector	 		10.0 m			R2

Rural/Industrial Collector		11.5 m			R3

Urban Local			8.0 m			R4

Rural/Industrial Local		8.0 m			R5

Lane				4.0-5.5m		R6

Urban Cul-de-sac Dia.		24 m 			R7

Urban Offset Cul-de-sac Dia.	See Drawings		R7

Rural/Industrial Cul-de-sac Dia.	28 m 			R8

Rural Offset Cul-de-sac Dia.	See Drawings		R8

7A.4	DESIGN CROSS SECTIONS

All road cross sections shall conform to the standard drawings unless otherwise approved by the Engineer.

7A.5	STREET CLASSIFICATION DEFINITIONS

“Downtown Street” means a street that serves traffic of regional importance and volumes of less than 5,000 veh. per day.  Downtown streets allow for direct access to adjacent land use.  The downtown streets are integrated with the collector street system and pedestrians and parking are a high priority.  It is not desirable to have truck and transit vehicles on downtown streets.

"Collector Street" means a street that serves traffic of sub-regional importance and volumes less than 5,000 veh. per day.  Collector streets collect traffic from the local road system, connect neighbourhoods within the Town and provide access to major trip destinations.  Pedestrians and cyclists are accommodated.

"Local Street" means a street that serves traffic of local importance and volumes less than 700 veh. per day.  Local streets are integrated with the collector street system and allow direct access to all adjacent land use.  Local streets integrate pedestrians and cyclists, but do not carry trucks or transit.

"Cul-de-sac" means a local street which has only one connection to the street system and which cannot reasonably be extended and which terminates in an area for the turning of motor vehicles.

7A.6	MINIMUM BASE AND PAVEMENT STRENGTHS

The minimum base and pavement strength standards shall be as shown below.  Proof of minimum strengths shall be required.

Lanes &

Urban Local	Other

Streets		Streets

Coarse Gravel Sub-base,

compacted 75 mm minus				250 mm		250 mm

Crushed Gravel Base,

compacted 20 mm minus				l00 mm		l00 mm

Hot plant-mix asphalt pavement,

compacted					50 mm		75 mm

Asphalt prime coat as directed

Maximum Benkelman Beam deflection		l.5 mm		0.75 mm

(when required by Engineer)

Minimum compaction of subgrade & gravel		95 %		95 %

modified	modified

proctor		proctor

7A.7	STANDARD LONGITUDINAL GRADE

7A.7.1	Minimum for all streets			0.50%

Maximum desirable

Collector and Local streets			8%

Maximum absolute

Collector & Local streets			12%

Maximum Arterial streets			8%

Maximum Cul-de-sac			6%

*	Maximum desirable approach

to an intersection				5%

*	Maximum desirable approach

to a T intersection			3%

Driveways up to 5m from street edge	12%

*  Unless otherwise approved by the Engineer

7A.7.2	Vertical Curvature at Intersection (K Values)

Providing the intersecting street is marked as a STOP, the following K Values may be used for the minor street:

Crest Curves		Sag Curves

Classification		Min	Pref		Min	Pref

Arterial			 4	 6		 4	 6

Others			 2	 4		 l.5	 4

Minimum K Values for sag curves may be used where street lighting is provided.

7A.8	STANDARD CROWN AND CROSSFALL GRADES

7A.8.l		Street pavements minimum		2%

Street pavements maximum		6%

7A.8.2		All roads shall normally be a crown section.  Crossfalls shall require prior approval from the Engineer.

7A.9	GEOMETRICS

Vertical curves, super elevation, spirals and intersection geometrics shall be in accordance with the TAC Manual.

7A.l0	EARTHWORK

7A.l0.l	Gravel and earthwork slope in cut and fill:

Desirable	2H:lV		Maximum l.5H:lV to be used only with

approval from the Engineer on a site

specific basis.

7A.l0.2	Rock slope in cut:

Maximum	lH:4V

7A.ll	STANDARD CROSS SECTIONS AND UTILITIES

The arrangements of utilities in relation to surface works and pavements and in relation to the right-of-way width shall be as shown in the Street Section Standard Drawings.

7A.12	CUL-DE-SACS

7A.12.1	Cul-de-sacs shall conform to the standard drawings.

7A.12.2	The length of any cul-de-sac shall not exceed 180 m to the end of the turnaround and shall permit a direct line of vision from the point of entry to the closed end.

7A.l3	TEMPORARY CUL-DE-SACS

Temporary cul-de-sacs shall be required for all temporary dead end streets.  Extent of works to be determined on a site specific basis and approved by the Engineer.

7A.l4	DRIVEWAYS

7A.l4.1	Residential driveways shall conform to Standard Drawing R10

7A.l4.2	Commercial driveways shall be reviewed by the Engineer on an individual basis.

7A.l4.3	A driveway permit must be obtained from the Municipality prior to construction of any driveway on an existing road.

7A.l5	STREET NAME AND TRAFFIC SIGNS

7A.l5.l	The design and construction of all roads shall include the design and installation of traffic signs (regulatory and warning), and traffic signals, in accordance with the most current standards contained in the "Manual of Uniform Traffic Control Devices for Canada" distributed by the Transportation Association of Canada (TAC).

7A.l5.2	Street name signs shall be provided at all intersections, where possible, the street name sign shall be located above a traffic sign at one corner of the intersection.

7A.l5.3	Sign locations shall be approved by the Engineer and in accordance with the most current standards contained in the "Manual of Uniform Traffic Control Devices for Canada" distributed by the Transportation Association of Canada (TAC).

7A.l6	STREET MARKINGS

7A.l6.l	The design and construction of all roads shall include the design and application of pavement markings and hazard and delineation markings in accordance with the most current standards contained in the Manual of Uniform Traffic Control Devices in Canada distributed by the Transportation Association of Canada (TAC).

7A.l6.2	Curb markings delineating "Bus Stops" shall be red, "No Parking" yellow, and "Loading Zones" white.

7A.l6.3	Crosswalk locations shall be determined as required by the Engineer.

7A.l7	COMMUNITY MAIL CENTRES

7A.17.1	Contact Canada Post to obtain agreement on the type and location of community and centralized mail centres (CMC).

7A.17.2	The location of the CMC will be marked on the drawings submitted for final subdivision approval.

7A.18	HALF ROAD

Amendment

Bylaw

1932

7A.18.1  The Approving Officer may allow the dedication and construction of a new half road provided that:

- The required minimum width of pavement for a half road is 6 metres, and the required minimum width of dedication is 10 metres;

- There is sufficient dedication to provide for two-way traffic flow and other infrastructure and services as required, such as sidewalk, street lights, and drainage collection system;

- All infrastructure and services must be contained within the 10 metre half road right-of-way.